JWG advice for an

Implementing regulation on interoperability requirements and non-discriminatory and transparent procedures for access to data required for demand response

Recitals to consider for inclusion

THE EUROPEAN COMMISSION,

Having regard to the Treaty on the Functioning of the European Union,

Having regard to Directive (EU) 2019/944 of the European Parliament and of the Council of 5 June 2019 on common rules for the internal market for electricity and amending Directive 2012/27/EU, and in particular Article 24(2) thereof,

Having regard to Directive (EU) 2024/1711 of the European Parliament and of the Council of 13 June 2024 amending Directives (EU) 2018/2001 and (EU) 2019/944 as regards improving the Union’s electricity market design

After consulting the Electricity Cross-Border Committee,

Whereas:

- Directive (EU) 2019/944 introduced several rules empowering consumers and providing them with the tools to participate in the energy markets. Notably, consumers shall be enabled to participate in demand response. In this process, the exchange of data required for demand response should be interoperable. To that end, Directive (EU) 2019/944 requires Member States to have due regard to the use of relevant available standards, including standards that enable interoperability on the level of the data model and the application layer, to best practices and the importance of the development of data exchange, to future and innovative energy services, to the deployment of smart grids and to the internal market for electricity.

- This Regulation is one of the implementing acts developed pursuant to Article 24(2) of Directive (EU) 2019/944  to set out interoperability requirements and non-discriminatory and transparent procedures for easy access to and exchange of data of the final customers referred to in Article 23(1) of Directive (EU) 2019/944 . The rules laid down in this Regulation are to facilitate the full interoperability of energy services within the Union in a harmonised way across Member States and increase the effectiveness of transactions that involve final customers data access and exchange by market participants, promote competition in the retail market and help avoid excessive administrative costs. The rules and recitals in this Regulation are in addition to the rules and recitals of the Commission Implementing Regulation (EU) 2023/1162 as well as the Commission Implementing Regulation (EU) 2025/XXX. This Regulation on final customers data required for  demand response should be read in conjunction with the Commission Regulation (EU) 2026/XXX (NCDR).

- To facilitate full data interoperability, this Regulation defines a reference model for the access to and exchange of data required for demand response and thus proposes a conceptual framework with standard roles, terminology, structure, components, and interactions. The reference model serves as an abstract blueprint for understanding, designing, and integrating demand response. Each role set out by the reference model is defined by a set of responsibilities and interactions. The reference model and more specifically the roles therein do not specify concrete implementation but outline general concepts and functions. Roles are assigned to entities at national level, allowing Member States to take national specificities into account.

- Roles and responsibilities defined in the Commission Implementing Regulation (EU) 2023/1162 and the Commission Implementing Regulation (EU) 2025/XXXX also apply to this Regulation where relevant. Market participants that exchange information following the specific procedures described in this Regulation should assume roles and responsibilities assigned by the reference model individually or jointly and may also carry out more than one role where appropriate. Therefore, it is necessary to specify the roles and responsibilities of market participants in the exchange of information pursuant to the reference model, specifically the roles described in Articles 5 to 23. In addition, a single role may be assumed by different entities for different situations.

- The processing of personal data exchanged using the procedures set out in this Regulation should comply with Regulation (EU) 2016/679 of the European Parliament and of the Council. Relevant eligible parties shall comply with their obligations under the relevant applicable EU rules.

- The arrangements for assigning and registering the Balance Responsible Party (BRP) for Service Providers, Service Provider Units (SPUs), or Service Provider Groups (SPGs) may vary across Member States. This Regulation provides a harmonised framework for data exchange and interoperability, while leaving the assignment of where and how to register the balance responsibility of a Service Provider to national regulation.

- The Directive (EU) 2019/944 contains several provisions for the development of demand response and an EU framework for demand response aggregation is put forward, incentivising Member States to encourage it. According to Directive (EU) 2019/944, final customers should have access to all organised markets and products, either directly or indirectly. The NCDR complements the existing EU regulatory framework and facilitates market participation of demand-response including load, energy storage and distributed generation, individually or aggregated, and facilitate market-based procurement of services by TSOs and DSOs.

- To simplify and streamline the qualification, verification, switching processes, and the relevant data exchange procedures, Articles 25 to 28 of NCDR propose a flexibility information system nationally implemented with a single or multiple SP modules and a single or multiple CU modules, and a single and common access point for all market participants. The registration of the controllable units (CUs) in the flexibility information system is required for the participation of CUs  in local or balancing services. The flexibility information system shall administrate and make available the CU master data, by means of providing one or several CU modules, and the  SPs, SPUs, SPGs data, by providing one or several SP modules. The reference model defined in the Annex to this Regulation includes the procedures for the registration and the distribution of the CU master data. It also applies for the SP registration, SPUs and SPGs registration, grid and product prequalification or verification, and the switching of market participant in accordance with Article 12 of Directive (EU) 2019/944. The reference model of this Regulation introduces two roles of CU module administrator and SP module administrator for the administration of CU and SP, SPUs, SPGs master data. It also foresees the role of CU registration responsible party with the responsibility to register the CU master data either directly (if the final customer is the CU registration responsible) or on behalf of the final customer (e.g. if the service provider or a Third Party is acting as a CU registration responsible).

- In line with Article 4 of Directive (EU) 2019/944 customers can have more than one metering and billing point (in this Regulation, the latter is further described as an accounting point) covered by a single connection point for their premises. Some connection points (with the grid) can be used to serve more than one customer premise. This Regulation addresses this by referring to a service validation point. One service validation point can comprise one or more accounting points. At the accounting point, the supplier and the balance responsible party are registered. The accounting point is already defined in Commission Implementing Regulation (EU) 2025/XXX.

- This Regulation applies to data exchange between final customers, market participants, including service providers and system operators. It lays down rules enabling final customers and entitled parties to exchange data required for demand response in a timely and secure manner. Upon obtaining relevant permission, the flexibility information system provides the relevant data to the eligible party chosen by the final customer or determined by national legislation. Moreover, the authorisation procedure could be linked to a contract or to an explicit clause within the contract between the final customer and the eligible party. In this way, access to data is ensured in a manner that respects consumer rights over their data, and market participants have a shared understanding of the type of data and workflows that are required for specific services and processes. Final customers can be required to confirm the allocated permission to different market participants such as service providers. Where a final customer terminates its  contract with a service provider or other market participant, the service provider or other market participant should continue to have access to the demand response related historical data necessary for billing or balancing purposes or dispute resolving. Member States may require certain demand response related data to be shared for legitimate public purposes, for instance with environmental or statistical authorities as well as with system operators or other market participants.

- This regulation refers to a role of entitled party as it is also the case in the Commission Implementing Regulation (EU) 2025/XXX. An entitled party must be involved in some data exchanges due to the responsibilities it exercises, and the tasks assigned to it. In this Regulation, the concerned responsibilities and specifics tasks are those defined in the national terms and conditions pursuant to Articles 11, 14, 24, and 45 of NCDR.

- In line with Article 16 of Directive (EU) 2019/944 and Article 22 of Directive (EU) 2018/2001 of the European Parliament and of the Council of 11 December 2018 on the promotion of the use of energy from renewable sources , as amended by Directive (EU) 2023/2413 of the European Parliament and of the Council of 18 October 2023 amending Directive (EU) 2018/2001, Regulation (EU) 2018/1999 and Directive 98/70/EC as regards the promotion of energy from renewable sources, and repealing Council Directive (EU) 2015/652, Member States shall ensure that citizen energy communities and renewable energy communities are able to access all suitable electricity markets, either directly or through aggregation, in a non-discriminatory manner. Citizen energy communities and renewable energy communities can access the balancing and local services markets by taking up the role of a service provider or by contracting a service provider on behalf of their members.

- As per Article 12(1) of Directive (EU) 2019/944, the switching of market participant engaged in aggregation by final consumers shall be carried out within the shortest possible time. The reference model in this Regulation enables the switching of aggregators in compliance with the requirements foreseen by Article 12(1) of Directive (EU) 2019/944.

- Under this Regulation, and to assist with the identification and the authentication of parties that are requesting access to data, Member States are recommended to support, as far as possible, digital solutions compliant with Regulation (EU) 2024/1183 (‘eIDAS Regulation’) to electronically identify and authenticate final customers and/or eligible parties. To this end, electricity undertakings should preferably rely on solutions that verify the identity using at least two authentication factors, which helps to reduce risks in fraud and identity theft. An example of electronic identification would be the European Digital Identity Wallets provided in accordance with European Digital Identity Framework established by Regulation (EU) No 910/2014. Cross border acceptance of European Digital Identity Wallets is already required under the provisions of Article 5 (f) (2) of Regulation (EU) No 910/2014 in the area of energy. Electricity undertakings can also rely on other electronic identification means that provide adequate assurance of the identity of the final customer or the involved party.

- The single and common access point of the flexibility information system can be established as a decentralised flexibility information system providing nationally harmonised access or as a centralised flexibility information system, if required by the national terms and conditions pursuant to Article 24 of NCDR. The data exchange shall be based on an ETSI-CEN-CENELEC set of standards  as defined in Article 26 of NCDR. More generally, NCDR foresees that entitled parties shall take into consideration ETSI-CEN-CENELEC standards and and other relevant European initiatives, such as the Harmonised Electricity Market Role Model (HEMRM) and the International Electrotechnical Commission’s Common Information Model (CIM).

- For the quantification, service validation and settlement of the delivered services, the NCDR foresees the implementation of baselining methods. A properly designed baselining method and calculation are important for the success of demand response, as it enables system operators and final customers to measure the performance of demand response resources. For the quantification and service validation, the calculated baseline is compared to metering data from smart meters or if foreseen in the Member State dedicated measurement devices (DMD)s, or calculated data based on a formula depicting the electricity injection or withdrawal behaviour for a period.  Having regards to Article 7b of Regulation (EU) 2019/943 of the European Parliament and of the Council, as amended by Regulation (EU) 2024/1747 of the European Parliament and of the Council and in accordance with the principles and rules defines in the NCDR, data from DMDs or calculated data can be used as a complement to validated historical metering data or near real-data from smart meters. For the purpose of quantification and service validation, the reference model of this Regulation introduces the roles of baseline calculator and administrator, and roles for the administration of the relevant data.

- The European Data Protection Supervisor was consulted in accordance with Article 42(1) of Regulation (EU) 2018/1725 of the European Parliament and of the Council and delivered an opinion on XX XX XXXX.

HAS ADOPTED THIS REGULATION:

CHAPTER 1 - SUBJECT MATTER AND DEFINITIONS

Article 1

Subject matter

- This Regulation lays down interoperability requirements and rules for non-discriminatory and transparent procedures for access to and exchange of data required for demand response in accordance with Articles 23 and 24 of Directive (EU) 2019/944. It also sets out non-discriminatory and transparent procedures for access to data that require reporting and publication of national practices applying the reference model.

- To ensure the application of interoperability requirements, this Regulation establishes a reference model for access to and exchange of data required for demand response. This reference model sets out the rules and procedures that Member States shall apply to enable data interoperability. The reference model identifies the electricity market participants that are affected by this Regulation and the roles and responsibilities they carry out individually or jointly as described in Chapter 2 Section 1 and in the Annex to this Regulation. A single electricity market participant may carry out multiple roles.

Article 2

Definitions

For the purposes of this Regulation, the following definitions shall apply:

- ‘demand response’ means demand response as defined in Article 2 of Directive (EU) 2019/944

- ‘flexibility information system’ means a flexibility information system as defined in Article 2 of NCDR

- ‘CU module’ means a CU module as defined in Article 2 of NCDR

- ‘SP module’ means a SP module as defined in Article 2 of NCDR

- ‘CU module administrator’ means a party responsible for storing and distributing CU master data within the flexibility information system

- ‘CU registration responsible party’ means a party entitled to interact with the CU module administrator to register, update and de-register CU master data on behalf of the final customer

- ‘SP module administrator’ means a party responsible for storing and distributing SP, SPUs and SPGs master data within the flexibility information system

- ‘service provider’ or ‘SP’ means a service provider as defined in Article 2 of NCDR

- ‘service providing unit’ or ‘SPU’ means a service providing unit as defined in NCDR

- ‘service providing group’ or ‘SPG’ means a service providing group as defined in NCDR

- ‘baseline’ means a baseline as defined in Article 2 of NCDR

- ‘baseline calculator’ means a party responsible for calculating the baseline of a resource according to a baselining method

- ‘baselining method’ means a baselining method as defined in Article 2 of NCDR

- ‘baseline administrator’ means a party responsible for storing and distributing baselines to entitled parties.

- ‘connecting system operator’ means a connecting system operator as defined in Article 2 of NCDR

- ‘procuring system operator’ means a procuring system operator as defined in Article 2 of NCDR

- ‘quantification data aggregator’ means a party responsible for the aggregation of validated historical metering and consumption data, near real-time metering and consumption data, DMD data or calculated data. This data is aggregated according to the terms and conditions pursuant to Article 11 of NCDR and pursuant to Article 18 of Commission Regulation (EU) 2017/2195.

- ‘quantification responsible party’ means a party responsible for calculating the difference between the baseline and the corresponding metering, measurement or calculated data for a specific resource and a specific timeframe.

- ‘dedicated measurement device data administrator’ or ‘DMD data administrator’ means a party responsible for storing and distributing dedicated measurement device data as defined in Article 7b of Regulation (EU) 2019/943, as amended by Regulation (EU) 2024/1747.

- ‘calculated data administrator’ means a party responsible for storing and distributing calculated data based on a formula depicting the electricity injection or withdrawal behaviour for a period.

- ‘near real-time metered data administrator’ means a party responsible for storing and distributing non-validated near real-time metering and consumption  data from smart meter or smart metering system as defined in Article 2(5) of Commission Implementing Regulation (EU) 2023/1162.

- ‘accounting point’ means an accounting point as defined in Article 2(3) of Commission Implementing Regulation (EU) 2025/XX

- ‘local service’ means a local service as defined in Article 2 of NCDR

- ‘balancing services’ as defined in Article 2(3) of Commission Regulation (EU) 2017/2195

- ‘CU master data’ is static data that defines a controllable unit and its characteristics. This data remains relatively constant or changes very rarely after it is collected and published.

- ‘entitled party’ is a party that must receive or send data because it is responsible for performing specific tasks according to the national terms and conditions pursuant to Articles 11, 14,24 and 45 of NCDR and pursuant to Article 18 of Commission Regulation (EU) 2017/2195.

- ‘permission administrator for demand response’ means a party responsible for administering data access permissions necessary for procedures defined in this Regulation and making the information about permissions available to final customers and eligible parties in the sector, on request;

- ‘temporary limit administrator’ means a party responsible for storing and distributing temporary limits on system elements, SPUs, SPGs or parts of SPGs to entitled parties.

- ‘product prequalification responsible party’ means a party  performing a product prequalification as described in the Articles 18 and 20 of NCDR and in accordance with the national terms and conditions for service providers pursuant to Article 11 of NCDR

- ‘product verification responsible party’ means party performing a product verification as described in the Articles 18 and 20 of NCDR and in accordance with the national terms and conditions for service providers pursuant to Article 11 of NCDR.

- ‘activation test responsible party’ means a party performing an activation test as described in the Articles 18, and 20 of NCDR and in accordance with the national terms and conditions for service providers pursuant to Article 11 of NCDR.

- ‘grid prequalification coordinator’ means a party responsible to coordinate a grid prequalification as described in the national terms and conditions for TSO-DSO and DSO-DSO coordination pursuant to Article 45 of NCDR

- ‘metered data administrator’ means metered data administrator as defines in Point (7) of Article 2 of Implementing Regulation (EU) 2023/1162

Chapter 2 -INTEROPERABILITY REQUIREMENTS AND TRANSPARENT AND NON-DISCRIMINATORY PROCEDURES FOR ACCESS TO DATA

# Section 1: Interoperability requirements - Reference model

Article 3

Implementation of the reference model

- Electricity undertakings in the retail electricity market shall apply the provisions set out in this Chapter and the reference model set out in the Annex.

- In order to meet the interoperability requirements, electricity undertakings that have implemented  provisions of NCDR and of national terms and conditions pursuant to Article 11, 14, 24 and 45 of NCDR shall apply the reference model set out in this Chapter and in the Annex of this Regulation for the elements, roles and procedures, related to the concerned provisions.

Article 4

Reference model and information on market organisation

- Member States shall report the national practices regarding the implementation of the interoperability requirements and procedures for access to data, required for demand response according to Article 23, and ensure that these comply with the obligations set out in this Regulation.

- Member States shall make the information on the organisation of the national market regarding specific roles and responsibilities set out in Table I of the Annex easily available to all eligible parties and final customers.

- The information referred to in paragraph 2 of this Article shall include the identification of those parties acting in the national market whose responsibilities are set out in Article 5 to Article 22 of this Regulation.

Article 5

Responsibilities of CU module administrator

- When the role of CU module administrator or SP module administrator are assigned to several market participants in a Member State, CU module administrator(s) shall cooperate closely with other CU module administrator(s) and SP module administrator(s) in its Member State to provide an online flexibility information system with a single and common access point as provisioned by Article 25 of NCDR and according to the provisions defined in the national terms and conditions for a flexibility information system pursuant to Article 24 of NCDR.

- . CU module administrator(s) shall ensure that access to CU master data is based upon validated permission from the final customer

- When transferring data to entitled parties, CU module administrator(s) shall  respect relevant personal data protection law and, where applicable, ensure, in cooperation with the permission administrator  that there is an active permission from the final customer or another legal basis for the final customers data to be lawfully transmitted or processed, including, where relevant, in accordance with Regulation (EU) 2016/679.

- To provide access to the relevant data and to make the relevant data available in a non-discriminatory and online manner to qualified service providers, from all Member States as provisioned by Article 27(a) of NCDR, who are qualified by the procuring system operator, the CU module administrator may use the identification and authentication services provisioned by Regulation (EU) 2024/1183.

- CU module administrator(s) shall:

- give eligible parties access to testing facilities where they can test the data exchange compatibility of their systems implementing the procedures defined in this Regulation;

- process changes to the list of entitled parties for each controllable unit;

- inform all entitled parties without unnecessary delay about changes of CU master data;

- keep a data access log for the final customers and make this information available to the final customer online, free of charge, without undue delay, and on their request; and

- make publicly available the relevant procedures they use as described by the reference model and illustrated in the Annex to this Regulation.

Article 6

Responsibilities of SP module administrator

- When the roles of CU module administrator or SP module administrator are assigned to several entities in a Member State, SP module administrator(s) shall cooperate closely with other SP module administrator(s) and CU module administrator(s) in the Member State to provide an online flexibility information system with a single and common access point as provisioned by Article 26(1)(a) of NCDR and according to the provisions defined in the national terms and conditions for a flexibility information system pursuant to Article 24 of NCDR.

- The SP module administrator may use the identification and authentication services provisioned by Regulation (EU) 2024/1183 to provide access to the relevant data and to make the relevant data available in a non-discriminatory and online manner to service providers from all Member States who are qualified by the procuring system operator as provisioned by Article 27(a) of NCDR.

- SP module administrator(s) shall

- ensure that all access to that data is based on a validated permission by the final customer.

- give eligible parties access to testing facilities where they can test the compatibility of their systems with the flexibility information system, including the single and common access point implementing the procedures defined in this Regulation.

- inform all entitled parties without unnecessary delay about changes to SPUs/SPGs data as described in the Annex to this Regulation.

- make publicly available the relevant procedures they use as described by the reference model and illustrated in the Annex to this Regulation.

Article 7

Responsibilities of service provider

- According to Article 5 of Regulation (EU) 2019/943 o, the service provider shall either be a balance responsible party or shall contractually delegate its responsibility to a balance responsible party of its choice.

- To ensure seamless demand response, the service provider shall:

- authenticate the final customer with whom there is a demand response contract;

- identify the related accounting point(s) and controllable units registered to the final customer who offers demand response;

- inform the final customer and obtain the relevant permission to perform all activities related to a contract involving demand response. In particular, the final customer must be well informed and have provided the relevant permissions related to the use of personal data by entitled parties linked to the participation in demand response;

- request registration in the CU module as the new service provider for the controllable units per specified date as agreed upon in new demand response contract with the final customer;

- request the update of the relevant data in the CU module and SP module without undue delay in case of change in the existing contract with a final customer or when the registered characteristics of the CUs or SPUs/SPGs change; and

- optionally, request to the CU module administrator the cancellation of the switching of service provider procedure, that already was initiated, when the final customer cancels the demand response service contract within the allowed regret period. The allowed regret period is defined at national level and without prejudice to Union rules on consumer protection.

Article 8

Responsibilities of CU registration responsible party

The CU registration responsible party shall:

- for the purpose of registration and updating of the CU master data in the CU module, authenticate the final customer and identify the CU.

- register and update the CU master data without any undue delay in case of change of the CU master data.

- collect data from the final customers to ensure a correct registration of the CU master data in the CU module(s) and therefore shall have correctly identified and informed the final customer of the registration process and have obtained the relevant permission from the final customer.

- ensure the veracity and the quality of the data exchanged with the flexibility information system.

- ensure that the final customers data be lawfully transmitted or processed in accordance with Regulation (EU) 2016/679. The CU registration responsible party shall verify that registered data of the CU is correct in the CU module.

Article 10

Responsibilities of baseline calculator

- The baseline calculator shall calculate the baseline according to the baselining method defined in the national terms and conditions for baselining methods pursuant to Article 14 of NCDR and send it to the baseline administrator, on its request, through an online interface or through another appropriate interface, on request, in a non-discriminatory way, without undue delay.

Article 11

Responsibilities of baseline administrator

- The baseline administrator shall

- make the baseline available to the quantification responsible party, on its request, through an online interface or through another appropriate interface, on request, in a non-discriminatory way, in accordance with the process defined at national level.

- give the eligible parties access to testing facilities where the eligible parties can test the compatibility of their systems with the systems of the baseline administrator implementing the procedures in this Regulation. The testing facility shall be available before the procedures are implemented and while they are in operation.

- For the duration of the retention period defined at national level, the necessary historical baseline data shall be kept available by the baseline administrator, along with the corresponding log information, for access by the relevant quantification responsible party and entitled parties on their request.

- Where Member States have so provided, a designated entity may take over the obligations of the baseline administrator referred to in paragraph 1(b).

Article 12

Responsibilities of metering point administrator

In addition to responsibilities established in Article 6 of Commission Implementing Regulation (EU) 2023/1162 the metering point administrator shall inform the CU module administrator and SP module administrator without undue delay, of any changes in the assignment of final customers to accounting points, and of any other external occurrences that invalidate active permissions granted in their area of responsibility.

Article 13

Responsibilities of quantification data aggregator

- The quantification data aggregator shall

  - receive or request the necessary information (e.g. calculation formula, list of involved controllable units) to perform the calculation of the metered data;

  - request from the relevant metered data administrator, DMD data administrator, calculated data administrator and near real-time metered data administrator the necessary data to calculate the metered data for quantification;

  - calculate the metered data for quantification according to a defined set of market rules; and

  - make available metered data for quantification to the quantification responsible party.

- The metered data for quantification referred to in paragraph 1 can be unitary or aggregated according to the national terms and conditions pursuant to Article 11 of NCDR or Article 18 of Commission Regulation (EU) 2017/2195.

Article 14

Responsibilities of calculated data administrator

The calculated data administrator shall:

- receive and store calculated data; and

- make calculated data available to quantification data aggregator, on its request, through an online interface or through another appropriate interface, on request, in a non-discriminatory way, and without undue delay.

Article 15

Responsibilities of dedicated measurement device data administrator

  - 1. The dedicated measurement device data administrator shall:

- Receive and store DMD data; and

- make DMD data available to the quantification data aggregator through an online or through another appropriate interface, on request, in a non-discriminatory way, and without undue delay. This data may be non-validated data or validated data, meaning data collected by a dedicated measurement device or completed with substitute values.

- To ensure seamless access to data for final customers and eligible parties, the dedicated measurement device data administrator shall:

  - make validated DMD data available to final customers and eligible parties through an online or through another appropriate interface, on request, in a non-discriminatory way, and without undue delay;

  - ensure that final customers (i) can access their validated DMD data; (ii) can make it available to eligible parties ensuring the consent of the final customer and (iii) receive it in a structured, commonly used, machine-readable and interoperable format;

  - keep a data access log up to date and make this available at the request of the final customer through an online or through another appropriate interface, free of charge, without unnecessary delay, and on final customer’s request;

  - when transferring data to eligible parties, and respecting relevant personal data protection law, ensure, in cooperation with the permission administrator where applicable, that there is an active permission or another legal basis for the data to be lawfully transmitted or processed, including, where relevant, in accordance with Regulation (EU) 2016/679 and in line with Commission Implementing Regulation (EU) 2023/1162; and

Article 16

Responsibilities of near real-time metered data administrator

- The near real-time metered data administrator shall:

- Receive and store near real-time metering and consumption data; and

- make the data available to quantification data aggregator through an online or through another appropriate interface, on request, in a non-discriminatory way, and without undue delay.

Article 17

Responsibilities of quantification responsible party

- The quantification responsible party shall perform the quantification in accordance with national terms and conditions pursuant to Article 11 of NCDR and to Article 18 of Commission Regulation (EU) 2017/2195.

- The quantification responsible party shall

  - Receive the information if quantification is needed for an activation

  - Request or receive the relevant period of activation from the entitled party

  - Request from the quantification data aggregator the necessary (aggregated) metered data for the related activation period

  - Request from the baseline administrator the necessary baseline data for the related activation period

  - Determine the relevant volume and make it available to entitled parties.

Article 18

Responsibilities of product prequalification responsible party

- The product prequalification responsible party shall perform the product prequalification according to Article20 of NCDR and national terms and conditions pursuant to Article 11of NCDR or Article 18 of EBGL.

- The product prequalification responsible party shall

- Notify the SP about the start of the product prequalification

- Request the activation test responsible party to perform an activation test if defined in the national terms and conditions for service providers

- Notify the entitled parties of the result of the product prequalification and confirm the SPU or SPG characteristics which are registered in the SP module.

Article 19

Responsibilities of product verification responsible party

- The product verification responsible party shall perform the product verification for the related product according to Article19 of NCDR and to the national terms and conditions pursuant to Article 11 of NCDR or Article 18 of EBGL

- The product verification responsible party shall notify the entitled parties of the result of the product verification and confirm the SPU or SPG characteristics which are registered in the SP module.

Article 20

Responsibilities of activation test responsible party

- The Activation test responsible party shall perform activation tests, in accordance with national terms and conditions pursuant to Article 11 of NCDR, and with Articles 19 and 20 of the NCDR.

Article 21

Responsibilities of temporary limit administrator

- The temporary limit administrator shall

- make the temporary limit available to the entitled parties, on their request, through an online interface or through another appropriate interface, in a non-discriminatory way, in accordance with the process defined at national level;

- Receive the temporary limits set or updated by the system operators.

- For the duration ofthe retention period defined at national level, the necessary historical temporary limits shall be kept available, along with the corresponding log information, for access by the relevant entitled parties on their request.

- Where Member States have so provided, a designated entity may take over the obligations of the temporary limit administrator referred to in paragraph 1(b).

Article 22

Responsibilities of grid prequalification coordinator

- The grid prequalification coordinator shall coordinate grid prequalification between the connecting and impacted system operators, in accordance with national terms and conditions pursuant to Article 45 of NCDR.

- The grid prequalification coordinator shall:

- Receive the request from the entitled party to coordinate Grid prequalification; and

- Notify the entitled parties of the result of the grid prequalification and the related status of the concerned SPU, SPG or part of SPG.

Article 23

Responsibilities of service acquiring responsible party

- The service acquiring responsible party shall

- Validate the message of bid received from the service provider

- Execute necessary actions to trigger for the selection of bid[s] and get the related results.

- Send the information to the service provider that its bid has been selected and the related information.

# Section 2: Transparent and non-discriminatory procedures for access to and exchange of data  –Reporting and repository of national practices

Article 23

Reporting of national practices

- In order to ensure the transparency and non-discrimination of national procedures for access to data required for demand response included in this Regulation, in line with Article 23 of Directive (EU) 2019/944, Member States shall

  - carry out and keep up to date a mapping of national practices at national level that also includes a detailed description and explanation of how the procedural steps of Tables III.1 to III.34 of the Annex to this Regulation are performed, by setting out which steps, if any, have been combined, and the order in which the steps are carried out; and

  - report to the Commission the mapping of national practices referred to in sub-paragraph (a) which shall be published in the publicly accessible repository established by ENTSO for Electricity and the EU DSO entity pursuant Article 12(1)(c) of Commission Implementing Regulation (EU) 2023/1162.

- The reporting of national practises shall include information on the national implementation of the reference model and of the various roles, information exchanges, and procedures.

- When a role or a procedure is not yet implemented according to Article 14 and 24 of NCDR, and to the national terms and conditions pursuant to Articles 11 of NCDR, Member states shall report on the implementation timelines. The obligation to include the mapping of a procedure in the report on the national implementation applies only XXX months after the implementation of the procedure.

- This reporting of national practises shall take into account the guidance developed by the Commission referred to in Article 25.

- Member States shall provide the reporting of national practices referred to in paragraph 1 to ENTSO-E and EU DSO Entity and ensure the  Commission is duly informed, by no later than on XX.XX.XXXX.

Article 24

Tasks under the EU DSO entity and ENTSO for Electricity cooperation on data transparency

- While preparing their advice and undertaking their supporting activities to the Commission, the ENTSO for Electricity and EU DSO entity shall cooperate closely with representatives of national regulatory authorities, competent authorities and regulated entities with institutional roles at national level regarding the right to access to demand response related data, as well as with all relevant stakeholders, including consumer associations, electricity retailers, European standardisation organisations, service and technology providers, and equipment and component manufacturers.

- In order to ensure transparency of access to data required for demand response across the European Union, the main tasks that ENTSO for Electricity and the EU DSO entity perform under their cooperation on data transparency shall include:

  - developing guidance to assist Member States in the reporting of national practices on demand response as set out in Article 25;

  - collecting the reports of national practices provided by Member States regarding the implementation of the reference model, as set out in Article 23; and

  - publishing and keeping up to date the reports of national practices provided by Member States in the publicly available repository established by ENTSO-E and EU DSO Entity pursuant to Article 12 (1)(c) of Commission Implementing Regulation (EU) 2023/1162.

- The ENTSO for Electricity and the EU DSO entity may also cooperate in assisting the Commission, following the process set out in paragraph 2 of Article 24, in the monitoring of the implementation of the reference model included in this Regulation and its further development as a result of regulatory, market or technology changes, building on the one developed for access to metering and consumption data pursuant to Commission Implementing Regulation (EU) 2023/1162

Article 25

Guidance for the reporting of national practices

No later than on XX.XX.XXXX, the Commission shall develop and make publicly available, assisted by the ENTSO for Electricity and the EU DSO entity, a guidance on the reporting of national practices on demand response.

CHAPTER 3 – FINAL PROVISIONS

Article 26

Entry into force and application

This Regulation shall enter into force on the twentieth day following that of its publication in the Official Journal of the European Union.

Article 3 shall apply XX.XX.XXXX.

This Regulation shall be binding in its entirety and directly applicable in all Member States.

OJ L 158, 14.6.2019, p. 125–199.

OJ L 2024/1711, 26.6.2024, p 1-18

Commission Implementing Regulation (EU) 2023/1162 of 6 June 2023 on interoperability requirements and non-discriminatory and transparent procedures for access to metering and consumption data (OJ L 154, 15.6.2023, p. 10–40).

Commission Implementing Regulation (EU) 2025/XXX of _ _ 2025 on interoperability requirements and non-discriminatory and transparent procedures for access to data required for customer switching (OJ XXX).

Commission Implementing Regulation (EU) 2026/xxxx of xx Month 202x establishing a Network Code on Demand Response (OJ XXX).

Regulation (EU) 2016/679 of the European Parliament and of the Council of 27 April 2016 on the protection of natural persons with regard to the processing of personal data and on the free movement of such data, and repealing Directive 95/46/EC (General Data Protection Regulation)

Directive (EU) 2018/2001 of the European Parliament and of the Council of 11 December 2018 on the promotion of the use of energy from renewable sources (recast); OJ L 328, 21.12.2018, p. 82-209.

Directive (EU) 2023/2413 of the European Parliament and of the Council of 18 October 2023 amending Directive (EU) 2018/2001, Regulation (EU) 2018/1999 and Directive 98/70/EC as regards the promotion of energy from renewable sources, and repealing Council Directive (EU) 2015/652; OJ L ,2023/2413, 31.10.2023.

Regulation (EU) 2024/1183 of the European Parliament and of the Council of 11 April 2024 amending Regulation (EU) No 910/2014 as regards establishing the European Digital Identity Framework; OJ L, 2024/1183, 30.4.2024.

Regulation (EU) No 910/2014 of the European Parliament and of the Council of 23 July 2014 on electronic identification and trust services for electronic transactions in the internal market and repealing Directive 1999/93/EC; OJ L 257, 28.8.2014, p. 73–114.

https://www.cencenelec.eu/news-events/news/2025/brief-news/2025-08-06-standards-framework-for-demand-response.

HEMRM - Harmonised Electricity Market Role Model  by ENTSO-E and EU DSO Entity, https://www.data-interoperability.eu/harmonised-electricity-market-role-model.

CIM - Common Information Model (IEC 61970, IEC 61968, IEC 62325 series, IEC 62746 series) https://www.iec.ch/homepage

Regulation (EU) 2019/943 of the European Parliament and of the Council of 5 June 2019 on the internal market for electricity (recast) (OJ L 158, 14.6.2019, p. 54-124).

Regulation (EU) 2024/1747 of the European Parliament and of the Council of 13 June 2024 amending Regulations (EU) 2019/942 and (EU) 2019/943 as regards improving the Union’s electricity market design (OJ L, 2024/1747, 26.6.2024).

Regulation (EU) 2018/1725 of the European Parliament and of the Council of 23 October 2018 on the

protection of natural persons with regard to the processing of personal data by the Union institutions,

bodies, offices and agencies and on the free movement of such data, and repealing Regulation (EC) No

45/2001 and Decision No 1247/2002/EC (OJ L 295, 21.11.2018, p. 39)